# AC Buzzer Project - GPIO Configuration Example

Your Air Conditioner Electricity Wastage Alarm

| GPIO Pin | Sensor/Component Name | Vcc | Ground | Function/Pins | Notes |
| --- | --- | --- | --- | --- | --- |
| GPIO4 | DHT22 | 3.3V | GND | Data pin (10kΩ pull-up to 3.3V) | Temperature/humidity sensor. Reads every 2 sec. Detects cooling slope when AC turns on. Uses Adafruit DHT library. |
| GPIO19 | Slide Switch (SPDT) | 3.3V | GND (via 10kΩ) | Common → GPIO19, Throw → 3.3V, Pull-down to GND | Arm/disarm switch. When ON: GPIO19 reads HIGH (3.3V). When OFF: pull-down resistor pulls to LOW (0V). |
| GPIO23 | Piezo Buzzer | 3.3V | GND | Signal Output (digitalWrite) | Passive buzzer driven directly from GPIO23. Sounds alert when AC has been cooling for 20+ minutes AND armed switch is ON. |
| — | Resistor 10kΩ (Pull-up) | 3.3V | GPIO4 row | One leg to 3.3V, other leg shared with DHT22 data | Keeps DHT22 data line HIGH by default. Sensor pulls it LOW to transmit. |
| — | Resistor 10kΩ (Pull-down) | — | GND | One leg to GPIO19 row, other leg to GND | Pulls GPIO19 LOW when switch is open. Prevents floating pin that would read garbage. |

## Logic Summary

The code reads the switch state (armed = HIGH, disarmed = LOW). If armed, it monitors temperature slope from DHT22. When temperature drops rapidly (AC cooling), a timer starts. If cooling persists for 20+ minutes, the buzzer sounds. Once AC turns off (temperature stabilizes), timer resets.